## LTC6812 Cell-Input Filter Options

This note compares the two proposed LTC6812 cell-input filter networks and their effect on open-wire detection.

The main issue is not normal cell-voltage measurement speed. The main issue is the LTC6812 open-wire check on the cell sense inputs.

The LTC6812 open-wire check uses the ADOW command. In this mode, the IC applies about 100 µA pull-up or pull-down current to the input and compares the measured voltage in the two cases. The datasheet method uses the difference between those two measurements, with a -400 mV threshold, to detect an open connection.

The external input resistor affects this directly because the ADOW current produces a voltage drop across it:

V = IR

For the two resistor values:

- 100 Ω→10 mV per pull direction, about 20 mV in the comparison

- 8 kΩ→0.8 V per pull direction, about 1.6 V in the comparison

That should be compared with the datasheet’s 400 mV threshold

The datasheet also gives the number of ADOW commands needed for different input capacitances:

- ≤10 nF: 2 ADOW commands

- 100 nF: 10 ADOW commands

- 1 μF: 100 ADOW commands in normal mode

So changing from 100 Ω/10 nF to 8 kΩ/1 μF changes both the voltage behaviour of the open-wire check and the number of ADOW cycles needed.

### Option 1 — 100 Ω/10 nF

- Keeps the input network close to the standard LTC6812 case

- Keeps the open-wire check close to the datasheet method

- No special threshold development work is expected

- Main limitation is that the resistor itself is not being used as the primary protection element in a severe local short case

### Option 2 — 8 kΩ/1 μF

- Reduces resistor stress in a severe local short case

- Pushes the open-wire check well outside the normal LTC6812 case

- The standard -400 mV threshold is no longer obviously valid

- A replacement threshold is not given in the datasheet and cannot just be assumed

- Any new threshold would have to be derived experimentally on the final hardware

- This is not a simple firmware adjustment; it becomes a custom diagnostic development and validation task

### Conclusion

Option 1 keeps the open-wire check close to the standard LTC6812 method.

Option 2 may improve resistor survivability in one fault case, but it makes open-wire detection a custom diagnostic that would need to be characterised and validated on the final hardware.